1. Explain Switch.

Ans:-

- A network switch is a device that connects multiple devices in a LAN and forwards data based on MAC addresses.

- Works at Layer 2 (Data Link Layer)

- Learns MAC addresses using a MAC table

- Sends data only to the intended device → improves performance

2. Explain Switch Boot Sequence.

Ans:-

- When a switch starts, it follows these steps:

- ) → checks hardware

- Boot Loader POST (Power-On Self Test loads → stored in ROM

- IOS loads → from Flash memory

- Configuration file loads → from NVRAM (startup-config)

- Switch becomes operational.

3.Expain Three Methods to Access Switch Command Line Interface.

Ans:-

- You can access switch command line using:

- Console Port

  - Direct cable connection

  - Used for initial setup

- Telnet

  - Remote access (not secure)

- SSH (Secure Shell)

  - Remote + encrypted (secure)

- Explain and Configuring the Cisco Internet Operating System.

Ans:-

- IOS (Internetwork Operating System) is the software that runs on Cisco devices.

Basic Configuration:

enable

configure terminal

hostname Switch1

enable secret password123

line console 0

password cisco

login

exit

interface vlan 1

ip address 192.168.1.1 255.255.255.0

no shutdown

- enable → privileged mode

- configure terminal → global config

- enable secret → encrypted password

- interface vlan 1 → management IP.

- Explain Switch Port.

Ans:-

- A switch port is a physical interface on a network switch used to connect devices like computers, printers, routers, or other switches.

🔹 Types of Switch Ports

1. Access Port

- Connects end devices (PC, laptop, printer)

- Belongs to only one VLAN

- Does not tag frames

- Example:

- interface fa0/1
switchport mode access
switchport access vlan 10

2. Trunk Port

- Connects switch-to-switch or switch-to-router

- Carries multiple VLANs

- Uses VLAN tagging (802.1Q)

Example:

- interface fa0/24

- switchport mode trunk.

6-R1, R2, R3, and R4 have their Fast Ethernet 0/0 interfaces attached to the same VLAN. A network engineer has typed a configuration for each router by using a word processor. He will later copy and paste the configuration into the routers. Examine the following exhibit, which lists configuration for the four routers, as typed by the network engineer. Assuming that all four routers can ping each other’s LAN IP addresses after the configuration has been applied, choose the routers that will be able to form a neighbor relationship with the other routers on the LAN. (You can assume that, if not shown in the exhibit, all other related parameters are still set to their defaults.) (Choose two)

A. R1

B. R2

C. R3

D. R4

E. None of the routers will exchange routing information.

Ans:- B. R2

7-enable secret [password] is hashed using the _________ algorithm.

A. MD5

B. AH

C. PSK

D. ESP

E. WPA2

Ans:-A.MD5.

8- An engineer connects to Router R1 and issues a show ip ospf neighbor command. The status of neighbor 2.2.2.2 lists FULL/BDR. What does the BDR mean?

A. R1 is an Area Border Router.

B. R1 is a backup designated router.

C. Router 2.2.2.2 is an Area Border Router.\

D. Router 2.2.2.2 is a backup designated router.

Ans:-D.Router 2.2.2.2 is a backup designated router.

9-Which command is used to view the neighbor discovery table on a PC?

A. show ipv6 neighbor.

B. show ipv6 neighbors.

C. netsh interface ipv6 show neighbor.

D. netsh interface ipv6 show neighbors.

Ans:-C.Netsh interface ipv6 show neighbor.

10-What type of variable is being shown? Routers = [R1,R2,R3]

A. List.

B. Dictionary.

C. Simple.

D. Unsigned integers.

Ans:-A.List.

11- Identify the fields in an IPv4 header. (Choose three)

A. Host component.

B. Time to Live.

C. Source address.

D. Destination address.

E. Network address.

Ans:-B.Time to live,C.Source address.,D.Destination address.